- IP Catalog -> search ILA, choose “ILA (Integrated Logic Analyzer)” -> Double-click

- Can choose various settings

  - Multiple probes depending on what information you want to look for

  - Larger sample data depth to collect more samples, generally a good thing but likely overkill after a certain point, depends on what you want to look for

  - On the Probe Ports tab, can choose how large for any particular probe width to be (if we want to take in a 32-bit signal for example)

- Hit ok after that and then generate on the next window that pops up -> wait some time for generation to finish

- After generation, need to actually use the ILA in one of our other files to incorporate it into the project

  - Use it wherever the data we want to view already exists!

  - Declare it as an instance of a module/component like the rest of our files

  - Should have clk as well as probe0 (probe1, probe2, etc. if using multiple probes) as the actual port names

  - Pass in the same clock used to generate the data you want to analyze as well as the data itself

- After this, do our normal synthesis -> implementation -> generate bitstream process

- Once we get to the program device part, there should now be a .ltx file in the line that says “Debug probes file” (or go find it in the same folder as your .bit file if not)

- Program the device as normal

- This should open up a new window similar to our simulation waveforms from before

- Can hit play to collect the stated number of samples for whatever information you passed in through the probe(s)

- Can also use the “Trigger setup” menu to capture a particular value for a particular probe bit or set of bits